Front Rotor

Resources:

Gen11 Brake Rotor Thermal Analysis

- Theoretical hand calcs of rotor temp increase

Avg h Calculation - Consolidated

- Check the methodology of calculating h… the rotor is spinning so its not just a regular flat plate in parallel flow

Revised -> Avg h Calculation

Copy of Gen XI Brakes DR 04/30/2026

Tachyon Brake Rotor Thermal Analysis

CAD

Solidworks files (with static sim setup and results):

https://github.com/ProtonKicker/Gen11-Brakes/tree/main/front%20rotor

Or download the entire repo (cad in the “front rotor” folder)

https://codeload.github.com/ProtonKicker/Gen11-Brakes/zip/refs/heads/main

Pack and go zip with sims:

https://drive.google.com/file/d/1uz7ue5qlSUVNbMzDQWOVXulgLMQuHuCY/view?usp=drive_link

Sims

Input parameters:

Static Sims

Setup:

- 325 N*m on brake pad (each wheel)

  - AI annotated calculations: https://github.com/ProtonKicker/Gen11-Brakes/blob/main/Brake_Calcs_Front_Rear_1g_Max_Braking.ipynb

- At road-tire interface, mu=1.0

- Tire radius: 0.279m

- Break pad positions offset by 15 deg

Results:

- FOS (min) = 1.2

- Weight: 1.64 lbs / 0.74 kg

- Apparently reversed direction has the same FOS for some reason

Position 1 (FOS 2.3)

Position 2 (FOS 1.3)

Position 3 (FOS 1.2)

Position 4 (FOS 1.2)

Position 5 (FOS 1.2)

Thermal Sims

Setups:

- Ambient Temperature: 310.93 K (100°F / 37.8°C )

- Pr = 0.707, h = 43.98482061

- Finest mesh

1g braking

Assumptions:

- 30 m/s speed, 1g slow down

- 183kg on the front

Result:

- 472.0 K / 198.85 °C / 389.93 °F

Notes:

- 0.05s interval, t = 3s

Temperature Distribution

Temperature vs Time

Continuous downhill braking

Setups:

- 25 m/s speed, 15 deg hill angle

- 1826.41N on the front or 913.205 N on each front wheel

- Radiation emissivity 0.8

Result:

- 1km at 30m/s until thermal shock threshold is reached

Temp distribution at t = 30s

Temperature VS Time

Repeated Duty Cycle

Setups:

- 0.2g accel when gaining speed

- 30m/s 1g braking

- 60s, 3 cycles

- Radiation emissivity 0.8

Heat Power (braking power)👇

Convection (h)👇

Results:

- 529.1 K / 255.95 °C / 492.71 °F

Temperature Distribution (Peak at 42.7s)

Temperature VS Time

Static Sims

Setup

- max braking force for front:  2327 N (at 1g weight transfer)

  - https://drive.google.com/file/d/18nv-dQ59la7i7q2jPzx00HYS4iSGQYc5/view?usp=drive_link for weight transfer calcs

  - At road-tire interface, mu=1.0

  - Tire radius: 0.279m

  - Torque = 650 N*m

Static 1

- Min FOS .49

Static 2

- Made it thicker (.1875”)

Min FOS: 1.06